      - MANUFACTURER

Specifier Notes: Specify Zurn Elkay Water Solutions

        - [Zurn Elkay Water Solutions, 1747 Commerce Way, Paso Robles, CA 93446. Toll Free (855) 663-9876.] Website: www.zurn.com

      - COUPLINGS

        - Basis of Design: Subject to compliance with requirements, provide Zurn; [Model FP23EZ1, One-Bolt coupling] or comparable product by one of the following:

            - <Insert manufacturer's name>

            - <Insert manufacturer's name>

            - <Insert manufacturer's name>

          - Standards: UL Listed, C-UL Listed, FM Approved

          - Pressure Rating: Maximum working pressure: 300 PSIG* (2068 kPa).

          - Size: NPS 1-1/4 to NPS 6 (DN 30 to DN 150).

          - End Connections: Grooved (AWWA C606)

          - Materials

            - Cast Coupling Housing: ASTM A536 65-45-12 Ductile Iron

            - Elastomers / Gasket: EPDM

            - Fasteners / Bolt & Nut: Grade 5

          - Standard Feature Options:

            - Easy-install three tab locating gasket

            - Pre-lubricated, pressure responsive gasket

            - Labor saving single-bolt design

- *Reference UL or FM listings for applications exceeding 300 PSIG (2068 kPa).